ORICK COMMUNITY SERVICES DISTRICT

EMERGENCY RESPONSE PLAN

WATER SYSTEM 1200701

Orick Community Services District

tABLE OF cONTENTS

O

rick is located in Humboldt County, CA approximately 45 miles north of Eureka. Orick is a rural town whose population of 466 is surrounded by Redwood National and State Parks and private forests.  Redwood Creek meanders through town and the agricultural lands, flowing into the Pacific Ocean.

Orick Community Services District (CSD) provides water and fire services to an area of approximately 2.3 square miles. The district recently undertook development of a community wastewater treatment and disposal project that is scheduled to be completed by summer of 2013.

Orick CSD was formed in 1955 for the purpose of providing community fire protection.  In 1976, the community authorized the district to develop a community water supply and distribution system.  Currently there are 140 active service connections including 102 residential, 6 multi-family residential, 28 commercial/industrial and 4 agricultural irrigation connections. The district also provides water services to the Redwood National Park Visitors’ Center.  In 2010, the district’s peak monthly use was 1.86 MGD. Approximately 73% of Orick households are serviced with community water.  Homes in the northern area of the community are not served with water as they are outside the CSD boundaries.

Water systems are vulnerable to natural and manmade disasters or emergency events.  Understanding a system’s vulnerabilities is an important part of emergency planning.  Recognizing this, Orick Community Services District has prepared this basic emergency response plan to act as a foundation for hazards that may be encountered. The type and needs of the incident will drive the level of response required to mitigate and correct the problem in a timely and effective manner.

Emergency or Disaster Type

Disasters can be broadly categorized as either Natural Disasters or Events Caused by Human Activities, and the severity of various types of events will drive the appropriate level of response.

Table 1

| Type of Event | Risk Type | Comments |
| --- | --- | --- |
| Natural Events |  |  |
| - Earthquakes | High |  |
| - Tsunami | High | Orick is recognized as a ‘Tsunami Ready Community’, with regular community drills. |
| - Floods | Low | Significant flood events have occurred, but construction of the Redwood Creek levee in 1967 has provided the community protection from floods. |
| - Waterborne Diseases | Low | Source water is from Redwood Creek, and there is minimal upstream development or source discharges which could introduce pathogens other than naturally occurring. |
| - Drought | Moderate | There is a need to plan and develop for additional sources and storage capacity. |
| Events Caused by Human Activities |  |  |
| - Levee Failure | Low | Levees were constructed by the US Corp of Engineers and maintained by the County of Humboldt. Levees are currently being evaluated for level of protection and certification (work on this section) |
| - Vandalism | Low | Water supply and distribution system has sufficient levels of protection. |
| - Terrorism | Low | CSD is in need of training on suspicious activity, response and protection of possible crime scenes. |
| - Cross-Connections/Backflow Conditions | Moderate | CSD needs to develop an inspection, repair and replacement program to minimize potential contamination. |
| - Chemical Spills | Low | Source water and wellhead is protected; any risk would be related to transportation of materials on Highway 101. |
| - Power Outage | Moderate | Defined as power outages lasting longer than 2 hours. OCSD does not have emergency generators and will be pursuing resources for long-term power outages. |
| - Sewage Spills | Low | OCSD is currently pursuing development of a community wastewater system, and this section will be revised at the appropriate time (est. Summer 2013) |
| - Fires | Low | Low probability, however fire suppression activities may tax existing system. Additional storage and backup generators is appropriate. |

Severity of Emergency

Understanding the severity of the emergency and being able to communicate effectively will help District employees and the Board to keep their response balanced, informed and effective when responding to the public, media and regulatory agencies.

The first step will be to assess the type and severity of an incident.  Once determined, notification of appropriate personnel is required.  Staff contact information is contained in Appendix 1.  Because the nature of emergencies and disasters can evolve or devolve over time the level of incident severity may change accordingly.

Level 1  - Routine Emergency

Defined as an event in which the water supply or distribution system experiences a pump failure, line-break or short-term power outage.  OCSD personnel are able to handle the problem with minimal outside assistance.  In this type of situation, it is unlikely that public health will be jeopardized, and OCSD personnel will be able to work and communicate effectively thorough the duration of the event.  Generally this type of event will be resolved within 24 hours.  These may include:

- Distribution Line Breaks

- Short Power Outages (lasting less than 4 hours)

- Minor Mechanical problems with the intake system or pumps

- Other minor situations in which public health is not likely to be jeopardized

Level 1 – Typical Response Activities

- Repair of line or equipment

- Pressure testing

- Disinfection

- Water sampling

If it is determined the problem may take longer than 24 hours to resolve, and water storage tanks are  likely to be drawn below a safe operating level, the situation will be elevated to a Level 2.

Level 2 – Minor Emergency

Defined as when the OCSD experiences a minor disruption in supply or has a possible contamination during which it may need to coordinate with California Department of Health and Human Services Office of Drinking Water, and consider issuing a health advisory to customers, community members and (where appropriate) the traveling public.  In this type of situation, public health may be jeopardized, consequently OCSD personnel will be on alert and initiate a quick response.  Minor emergencies can usually be resolved within 72 hours.  These may include:

- Disruption in supply, such as a transmission main line break, pump failure with a potential for backflow, and loss of pressure

- Inadequate storage to handle disruption in supply

- A water sample with an initial positive result for coliform or E.coliform

- An initial primary chemical contaminant sample

- A disruption in chlorine/chemical disinfection feed

- A minor act of vandalism (with the exclusion of graffiti/tagging type activities) that could have exposed water system to having been tampered with

- Drought with a noticeable and continuing decline of water level

Level 2 – Typical Level of Response

- Repair of line or equipment

- Pressure testing

- Disinfection

- Back up water sampling in accordance with Sampling Plan

Level 3 – Significant Emergency

Is defined as where the OCSD water system experiences major damage or contamination from a natural disaster, accident or an act of terrorism.  These incidents will likely require immediate notification of local law enforcement, fire department, local public health officer, and the local Office of Emergency Services.  Significant emergencies should be reported to CDHS-ODW as soon as possible to determine the best available means to protect public health.  OCSD personnel are directed to the event, and outside entities are notified to aid in response.  Major emergencies may require more than 72 hours to resolve.

- A water sample with a confirmed positive coliform or E. coliform requiring immediate consideration of a health advisory notice to customers

- A water sample with another confirmed primary contaminant requiring immediate consideration of a health advisory notice to customers

- A loss or complete malfunction of the water treatment system for the surface water source, including chlorination

- A major line break or other distribution system failure resulting in water shortage or requiring system shutdown

- An act of vandalism or terrorist threat, such as intrusion or damage to the water treatment facilities

- Fire – either one that destroys water treatment facilities or suppression demand that exceeds the system capacity to provide water

- Severe drought that significantly affects water withdrawal abilities

Due to OCSD’s limited personnel and resources, it may be necessary to utilize Mutual Aid Agreements.

Level 4 – Catastrophic Disaster/Major Emergency

Is defined as where the OCSD experiences major damage or contamination from a natural disaster, accident or an act of terrorism.  These incidents will require immediate notification to local law enforcement, public health officer, emergency responders, and local Office of Emergency Services.  Immediate issuance of health advisories and declaration of water supply emergencies are critical to protection of the public’s health.  These events may take several days or weeks to resolve before the system returns to normal operations.

- Earthquake that shuts down the water system or impacts distribution lines or storage tanks

- Flood that infiltrates system facilities and sources

- Levee failure resulting in flooding

- Storm that significantly damages power transmission lines, grid or system facilities

- Hazardous chemical spill within 2,000 feet of the water systems source

- Act of terrorism possibly contaminating the water system with biological or chemical agents

Due to OCSD’s limited personnel and resources, it may be necessary to utilize Mutual Aid Agreements.

Attachment 1

OCSD Emergency Contact Information

Orick Community Services District

Director and Personnel Contact Information

Table 2

| Name | Position | Phone | Cell | Email |
| --- | --- | --- | --- | --- |
| Ron Barlow | President | 707-488-5952 | 707-496-4192 |  |
| Robert Secor | V.President |  | 707-498-2907 |  |
| Byron Frick | Board Director | 707-488-9512 |  |  |
| Marla Zuber | Board Director |  |  |  |
| Kaitlyn Combs | Board Director | 707-488-2177 |  |  |
| Bill J. Allen | Water Maintenance | 707-488-2212 | 707-498-2411 |  |
| Trevor Avram | Water Operator Maintenance | 707-488-5741 | 530-520-2744 |  |
| Barbara Mitchell | Administrative staff and Book-keeping | 707-488-3042 | 707-496-1498 |  |
| Marcie Allen | Office Staff | 707-488-2212 | 707-498-2452 |  |

Orick Volunteer Fire Department

Response Personnel

Table 3

| Name | Position | Phone | Cell | Pager |
| --- | --- | --- | --- | --- |
| Steven Baker | Fire Chief | 707-488-2271 | 707-498-4637 |  |
| Tim Duncan | Volunteer |  | 707-845-1380 |  |

Attachment 2

Orick Community Services District

Water System Information

OCSD Water System Information

In the event of an emergency, both OCSD personnel and California Department of Health and Human Services – Office of Drinking Water staff may need to have the following information readily available.

Table 4

| Orick Community Services District |  |  |
| --- | --- | --- |
| System Identification Number: | DHS-ODW Water System: SWRCB Permit Number: |  |
| Name and Address | Orick Community Services District PO Box 244 101 Swan Road Orick, CA 95555 |  |
| Directions to Office | Highway 101 north to Orick, turn right at the Orick Market, turn left and follow to end of lane |  |
| Number of Service Connections and Population Served (2010) | 109 Residential Connections 4 Multi-family residential 25 Commercial/retail 5 Agricultural |  |
| Type of Source Water | Groundwater intake from Redwood Creek |  |
| Type of Disinfection/Treatment | Chlorination |  |
| Number of Storage Tanks | Two (250,000 gallons each) |  |
| Average Water Demand (2010) | (Total delivered) (Average Monthly) (Average Daily) (Average Hour) |  |
| Maximum and Peak Water Demand |  |  |
| Emergency Contact Ron Barlow Bill Allen Trevor Avram Joe Hufford | Board President Water Maintenance Water Operator Contractor | 707-496-4192 707-498-2411 530-520-2744 707-488-2602 |

Attachment 3

OCSD Boundary Map

&

Water System Map

Attachment 4

Emergency Resources

Attachment 5

Communication Procedures

(Internal & External)

Attachment 6

Public Notice Procedures

Attachment 7

Contact List

Internal

External

Media

OCSD (Internal) Contact List

| Name | Position | Phone | Cell Phone |
| --- | --- | --- | --- |
| Ron Barlow | Board President |  |  |
| Robert Secor | V. President |  |  |
| Byron Frick | Director |  |  |
| Marla Zuber | Director |  |  |
| Kaitlyn Combs | Director |  |  |
| Bill Allen | Water Maintenance |  |  |
| Connie Secor | Office/Book- keeper |  |  |
| Marcie Allen | Office Staff |  |  |
| Trevor Avram | Water Operator |  |  |
| Fire Department |  |  |  |
| Steven Baker |  |  |  |
| Tim Duncan |  |  |  |

External Contact List

| Name | Position | Phone | Email |
| --- | --- | --- | --- |

OCSD Water System Emergency Notification List

Media

| Television | Location |  | Phone | Email |
| --- | --- | --- | --- | --- |
| KIEM |  | Eureka | (707) 443-3123 |  |
| KVIQ |  | Eureka | (707) 443-6666 |  |
| KEET |  | Eureka | (707) 445-0813 |  |
| Radio |  |  |  |  |
| KATA |  |  | (707) |  |
| KFMI |  |  | (707) |  |
| KHSU |  |  | (707) |  |
| KEKA |  |  | (707) |  |
| KINS |  |  | (707) |  |
| KRED |  |  | (707) |  |
| KSLG |  |  | (707) |  |
| Newspapers |  |  |  |  |
| Times Standard |  | Eureka/Daily | (707) 441-0500 | kwear@times-standard.com |
| North Coast Journal |  | Eureka/Weekly | (707) 442-1400 | newsroom@northcoastjournal.com |
| McKinleyville Press |  | McKinleyville/Weekly | (707) 839-0795 | news@mckinleyvillepress.com |
| Daily Triplicate |  | Crescent City/Daily | (707) 464-2141 | rwiens@triplicate.com |

Attachment 8

Water Quality Sampling Plan

&

Backup Sampling Plan